| Variable | Type | Description |
| --- | --- | --- |
| T701 | Temperature sensor | Reboiler heating-jacket temperature; important heat-source information |
| T702–T712 | Temperature sensors | Core process-temperature information |
| P701 | Pressure sensor | Vacuum/process pressure |
| P702 | Pressure sensor | Vacuum/process pressure |
| PDI701 | Differential pressure sensor | Pressure drop along column; potentially highly informative |
| PDI702 | Pressure sensor | Static pressure in buffer vessel V002 |
| PY23 | Pressure sensor | Top-vessel/exhaust pressure |
| FT703 | Flow sensor | Process flow information |
| FT704 | Flow sensor | Process flow information |
| FYI702 | Flow sensor | Cooling-water mass flow, L/s |
| LS701 | Level sensor | Reaction-vessel level |
| LS702 | Level sensor | Buffer-vessel level |
| H002 | Heating actuator | Reboiler heating-rod power |
| H701 | Heating actuator | Reboiler-base heating |
| H702 | Heating actuator | Auxiliary heating |
| H704 | Heating actuator | Auxiliary column heating |
| H706 | Heating actuator | Auxiliary column heating |
| H708 | Heating actuator | Auxiliary column heating |
| P301 | Vacuum-pump actuator | Vacuum-generation state |
| TV1 | Vacuum/bleed valve | Controls vacuum via ambient-air admission |
| AV709 | Nitrogen valve | Inertisation state; may be mostly constant but useful as process context |
| AV716 | Cooling valve | Head-condenser cooling |
| AV8 | Valve/status | Only 2 states and ~97.4% identical; currently little apparent information |
| AV709 | Nitrogen valve | Process-state information despite being mostly constant |
| AV716 | Cooling valve | Cooling-system state |
| P301 | Vacuum pump | Vacuum-system operating state |

| Feature | Formula | Description / use in analysis |
| --- | --- | --- |
| T703_mean | x=160i=160xi | Average temperature in the 60-s window; captures the process state/level. |
| T703_std | s=159i=160(xi-x)2 | Within-window variability; identifies unstable or fluctuating behaviour. |
| T703_min | min⁡x1…x60 | Lowest temperature; captures extreme low values. |
| T703_max | max⁡x1…x60 | Highest temperature; captures extreme high values. |
| T703_slope | xi=a+b i, feature = b | Overall trend within 60 s; detects heating/cooling or abnormal drift. |
| T703_mean_prev | xt-1 | Mean temperature in the previous 60-s window; provides temporal context. |
| T703_mean_delta | xt-xt-1 | Change between consecutive windows; detects abrupt or unusual process changes. |
| T703_std_delta | st-st-1 | Change in variability; detects transitions toward unstable behaviour. |
| T703_slope_delta | bt-bt-1 | Change in temperature trend; detects changes in process dynamics. |
| T703_T709_corr | r703,709=cov⁡T703T709σ703σ709 | Relationship between temperatures within the window; detects abnormal process coupling. |
| T703_T709_diff_mean | mean⁡T703-T709 | Average temperature difference; captures the temperature gradient between process locations. |
| T703_T709_diff_std | std⁡T703-T709 | Variability of the temperature difference; detects unstable gradients. |
| T703_T709_diff _slope | slope of T703-T709 | Change in temperature gradient; detects changing process relationships. |
| total_power_T703 | kPk | Overall signal fluctuation in the frequency domain; detects increased oscillatory activity. |
| dominant_power _ratio_T703 | max⁡PkkPk | Fraction of power in the strongest frequency; identifies pronounced periodic behaviour. |
| spectral_entropy _T703 | -kpklog2pk, pk=Pk/Pk | Distribution of spectral power; distinguishes structured/regular vs. irregular fluctuations. |

Pk=∣Xk∣2N
where:

- Xk= FFT result at frequency bin k

- N=60= number of samples

| Set | Groups (batch, operating_point) | Experiments |
| --- | --- | --- |
| Training | — | 38 normal |
| Validation | 20 | 39 observable-anomaly |
| Final test | 20 | 40 observable-anomaly |
| Unobservable faults | — | 2 excluded |

|  | Validation | Final test |
| --- | --- | --- |
| Experiments | 39 | 40 |
| Groups | 20 | 20 |
| Windows | 223,961 | 237,035 |
| Observable-anomaly windows | 65,143 | 70,952 |
| Observable-anomaly rate | 29.1% | 29.9% |

Baseline (IF with window features) and 1st model (IF with temporal features)

Feature representation

- Baseline features: ROC-AUC 0.692, AP 0.504

- Temporal features: ROC-AUC 0.730, AP 0.562

- → Temporal features preferred

Model

- Isolation Forest successfully ranks anomalous windows above normal windows.

- Temporal features improve this ranking.

- Default threshold is not appropriate for the desired recall.

- A training-calibrated 90th-percentile threshold gives F1 ≈ 0.547 on the held-out test set.

Scientific interpretation

- Normal → blind → observable anomaly/recovery shows a progressive increase in anomaly score.

- Blind phase remains difficult to distinguish from normal.

- Observable anomaly and recovery are substantially easier to detect.

- This supports the hypothesis that temporal process changes are informative for anomaly detection.

Evaluation setup

Primary metric:

- AUPRC (Area Under the Precision–Recall Curve)

  - Appropriate because anomalies are relatively rare.

  - More informative than accuracy and generally preferable to ROC-AUC for an imbalanced anomaly-detection problem.

- F1 score (secondary metric)

Methods reported in the publication:

| Method | Type |
| --- | --- |
| DONUT | VAE-based time-series anomaly detection |
| GMM-VAE | VAE + Gaussian mixture model |
| USAD | Unsupervised reconstruction-based AD |
| LSTM-VAE-GAN | LSTM + VAE + GAN |
| TADGAN | Time-series GAN |
| MADGAN | Multi-generator adversarial model |
| THOC | Hierarchical temporal representation / contrastive AD |

PowerPoint

From Process Data to Fault Diagnosis

Explainable Anomaly Detection for Batch Distillation

GitHub repository

batch-distillation-anomaly-detection

README title:

From Process Data to Fault Diagnosis

Explainable Anomaly Detection for Batch Distillation

Streamlit

Process Fault Diagnosis

Batch Distillation Anomaly Detection

Phase A — Model family comparison

Baseline temporal features

- Isolation Forest

- LOF

- PCA reconstruction

Compare:

- ROC-AUC

- Average Precision

- score distributions by label

- computational complexity

- stability

Phase B — Frequency-domain experiment

Investigate the raw 60-second windows:

- sampling interval

- frequency resolution

- dominant frequencies

- whether normal vs anomaly windows actually differ spectrally

If there is meaningful spectral information:

Temporal features + Fourier/periodogram features

→ train the best-performing anomaly detector again.

Phase C — Final model

Only then:

- threshold calibration

- anomaly detection

- feature/variable explanation

- process-area diagnosis

This keeps the project scientifically clean:

Does temporal information help? → Does another anomaly-detection principle help? → Does frequency-domain information add information? → Which model gives the best explainable result?